# Line Maze Solver RC Bot Project Report

## 1. Introduction

This project presents the design and implementation of an RC-based Line Maze Solver Bot. 
The bot is capable of following a black line on a white surface and can solve maze-like paths autonomously. 
It combines manual RC control features with automatic line following functionality for enhanced flexibility.

## 2. Objective

The objective of this project is to develop a robot capable of autonomously navigating a maze using line-following sensors, 
while also being controllable via a radio control system. The project demonstrates a blend of autonomous robotics and remote control systems.

## 3. Components Used

- Arduino UNO (Microcontroller)
- 8-Channel IR Line Sensor (RLS08)
- L298N Motor Driver Module
- N20 Motors (1 Pair)
- Buck Converter (for 5V regulation)
- RC Receiver and Transmitter (2.4GHz)
- 12V Li-ion Battery Pack
- Chassis with wheels

## 4. Circuit Description

The IR sensor module detects the black line using infrared reflectance. 
The output of each sensor segment is connected to Arduino’s digital input pins.
The Arduino processes these signals and sends appropriate control commands to the L298N motor driver to move the robot.
The buck converter provides regulated 5V to the Arduino and sensor module. 
The RC receiver allows manual control, and a mode switch toggles between RC and Autonomous modes.

## 5. Working Principle

The bot uses infrared sensors to detect the path and stay aligned with the black line. 
Each IR sensor detects light reflectivity from the surface. 
When the black line is detected, the sensor outputs a LOW signal, while the white surface gives a HIGH signal.
Depending on which sensors are active, the Arduino adjusts motor speed and direction to keep the bot on track.

## 6. Applications

- Maze-solving competitions and robotics contests
- Autonomous delivery and inspection robots
- Educational robotics projects
- Prototyping autonomous navigation systems

## 7. Conclusion

The Line Maze Solver RC Bot successfully integrates autonomous navigation with manual RC control. 
It demonstrates practical applications of sensor-based decision-making, motor control, and embedded systems. 
This project can be further enhanced with features like obstacle detection, camera integration, or wireless telemetry.